# Annex II – Core TSOs general measures and action plan to avoid future cross-zonal capacity reductions

Q2 2022

Table of Contents

Introduction	3

German TSOs	3

APG	4

ELIA	6

ELES	7

HOPS	9

PSE	10

RTE	11

SEPS	13

Transelectrica	14

TENNET TSO BV	18

# Introduction

According to Articles 20(14)(b) and 20(15) of the DA CCM, Core TSOs have the obligation to provide general measures and/or action plans in order to avoid cross-zonal capacity reductions in the future, as follows:

- As per Article 20(14)(b): General measures to avoid cross-zonal capacity reductions in the future

- As per Article 20(15): When a given Core TSO reduces capacity for its CNECs in more than 1% of DA CC MTUs of the analysed quarter, the concerned TSO shall provide to the CCC a detailed report and action plan describing how such deviations are expected to be alleviated and solved in the future.

This annex contains the required information described above for each Core TSO that has applied capacity reductions for at least 1 DA CC MTU of the analysed quarter.

# German TSOs

In order to provide a better understanding of the results and the processes applied among German TSOs, the following are some introductory explanations:

- APG, Tennet NL and the German TSOs (DAVinCy TSOs) use a common tool for individual validation called DAVinCy (Day Ahead Validation of Capacity). In case an overload cannot be solved with the available remedial actions, DAVinCy determines the necessary Individual Validation Adjustments (IVAs) on relevant CNECs with the objective to minimise the overall capacity reduction among the six DAVinCy TSOs. This can lead to situations where an overload occurs in one control area of TSO A whereas the IVA(s) is/are applied within other control areas, e.g. of TSOs B and C.

- The application of an IVA prevents a network element from being overloaded and generally does not lead to a deviation from the minimum capacity according to Article 16(8) of regulation (EU) 2019/943 on the internal market of electricity. In case of a deviation from the minimum capacity, the compliance with the minimum capacity targets of the German action plan according to the monitoring methodology approved by the German NRA Bundesnetzagentur will be detailed within the German Compliance Monitoring Report for 2022 that will be finished in mid-2023[1].

- The short time period covered by this IVA report (09.06.2022 – 30.6.2022) does not allow for general conclusions or interpretation of the observed data. More information is expected when a broader data basis is available.

- The subsequent report for Q3 is expected to include more substantial information.

The DAVinCy TSOs plan to apply the following measures in order to minimize the application of IVAs and, as required by art 20(14)(b) and 20(15) DA CCM, avoid cross-zonal capacity reductions in the future:

- Improvements within the DAVinCy tool such as cross-border redispatch between Germany and the Netherlands (already implemented in the tool, will be activated after clarification of details of the operational process) and the application of additional topological measures. Further improvements are foreseen to consider redispatch between DAVinCy TSOs’ control areas and non-DAVinCy TSOs’ control areas if redispatch potential is securely available and consideration of countertrade between Germany and Denmark.

- Continuous improvements of the input data: Many of the overloads that caused IVA application in all DAVinCy TSOs’ grids where due to erroneous data from Coreso related to AT-IT cross-border flows in the net-position forecast. This error has been fixed and measures are taken to prevent IVA application when similar problems arise.

- Improvement of process robustness through the use of redundant IT systems, plausibility checks and replacement strategies for input data, separate systems for test- and productive environment and implementation of security management.

- The recent relaxation of the fallback will lead to lower IVA in case of a failure of the DAVinCy process.

- The planned grid enforcements within the control areas of the German TSOs will relieve grid elements with overloads in the future (cf. Action Plan of Germany[2] and Network Development plan[3]).

[1] See: approved Monitoring Report (2022)

[2] See: Chapter 2 German Action Plan

[3] See; https://www.netzentwicklungsplan.de/en/front

# APG

In order to provide better understanding of the results and the processes applied among APG, Tennet NL and German TSOs following some introductory explanations:

- APG, Tennet NL and the German TSOs (DAVinCy TSOs) use a common tool for individual validation called DAVinCy (Day Ahead Validation of Capacity). In case an overload cannot be solved with the available remedial actions determines the necessary IVAs with the goal of minimising the overall capacity reduction for the six DAVinCy TSOs caused by the IVAs. This can lead to situations where an overload occurs in one control area of TSO A whereas the IVAs are applied within other control areas e.g. of TSO B.

- The application of an IVA prevents a network element from being overloaded and does not necessarily lead to a violation of minimum capacity levels according to the Clean Energy package.

- The short time period covered by this IVA report (9.6.2022 – 30.6.2022) is lacking general conclusion or interpretation of the observed data. More information is expected, when a broader data basis is available.

- The following report for Q3 is expected to include more substantial information.

The DAVinCy TSOs plan to apply the following measure to reduce the application of IVAs in the future:

- Improvements within the DAVinCy tool such as the application of additional topological measures, cross-border redispatch between Germany and the Netherlands as well as congestion management measures between the control areas of DAVinCy and non-DAVinCy TSOs e.g. countertrade between Germany and Denmark.

- Continuous improvements of the Input data: Many of the overloads that caused IVA application on CNECs of DAVinCy TSOs were due to erroneous data from Coreso related to AT-IT cross-border flows in the net-position forecast. This error has been fixed and measures have been taken to prevent IVA application when similar problems arise.

- Improvement of process robustness through the use of redundant IT systems, plausibility checks and replacement strategies for input data, separate systems for test- and productive environment and implementation of security management.

In addition to improvements of the individual validation process and DAVinCy itself, APG plans to implement further measures according to the Austrian Action Plan. Those measures include, but are not limited to:

- Network reinforcement and optimisation, e.g. dynamic line rating to relieve grid elements with overloads in the future

- Network expansion and planned infrastructure projects according to the Network Development Plan

# ELIA

General measures include, but are not limited to:

- A daily follow up plus short-term mitigations and long-term local validation of the tool has been set up to improve the tool aiming to:

  - reduce the number of occurrences of fallbacks,

  - reduce the number of occurrences of spanning by switching to DCLF when no ACLF converges,

  - reduce the number of occurrences of excessive IVA application due to failure in the derogation step.

- Congestion management through the shifting of overloads from the congested element to other CNECs via PST optimization to reduce the overall overloads, this action is already in place.

- Common Core action ongoing to correct the DCimbalance corrected and the CGM used in the FB DA CC process to make the flowbased calculations more correct.

Temporary structural situation:

- The nuclear phase out planned in Belgium is going to decrease the congestion around Doel and Mercator which are the most congested area. There is no additional action plan concerning these IVAs.

Situation linked to short outages:

- There are no extra investments foreseen to cover N-2 situations like for temporary short outages.

# ELES

General measures to avoid cross-zonal capacity reductions in the future, as per Art. 20(14)(b) of DA CCM

- Improve reliability of the local validation tool – at several occasions, IVA was applied as a part of fallback process due to local validation tool failure. By improving the reliability of local tooling, fallback process and thus application of IVA is avoided.

- Additional training of the operators – to improve reliability of the validation process

- Improvement in congestion management – we will continue to improve the quality of our inputs for CORE DA CC in order to avoid unnecessary IVA application in case of errors in inputs files.

- Network development and optimisation

Detailed report and action plan describing how such deviations are expected to be alleviated and solved in the future

In the first quarters after CORE DA CC go-live, ELES applied reduction for more that 1% of MTU. There are three main reasons for this:

- Local validation tool failure: on two occasions (18.6. and 19.6.), we faced a local validation tool failure, that lead us to apply the fallback procedure that conservatively apply bulk reductions in order to ensure grid security.

- Uncertainty level of our internal validation tool – our validation tool includes an option to set a reliability margin in order to strongly increase confidence to provide safe operation of the grid in real time. Due to the fact that CORE DA CC was a new process for our operators, the reliability margin was set conservatively. After some operational experience, the reliability margin will be lowered.

- Network weaknesses – there are some weaknesses on our network, most critical are the lines Podlog – Obersielach (SI-AT) and Divaca – Pehlin (SI - HR).

Our plan to improve the situation consists of the following:

- Additional training of operators in order to improve the process and to avoid local tool failures due to operators’ error

- Cooperation with the vendor of the validation tool to improve stability and reliability of the tool

- Analysis will be performed on accuracy of validation tool (e.g. comparing the flows considered during the validation and realised flows). Based on the result of the analysis, the validation tool reliability margin will be adjusted in order to decrease the level of IVA application.

- Network development and optimisation

# HOPS

General measures include, but are not limited to:

- Network development and optimisation

The goal is to increase the transmission capacity and reduce grid congestion. The measures to achieve these goals include strengthening the optimising the existing network and the development of new infrastructure.

- Improvements concerning congestion management

Core CCR coordinated improvements based on shared forecasts and aligned assumptions in capacity calculations with coordinated actions to increase cross-zonal capacities and reduce uncertainties (CGM improvements, Coordinated validation, etc.). Also, inclusion of third countries could open further opportunities for HOPS (with planning process and implementation of remedial measures). Unscheduled allocated flows coming from commercial exchanges outside the Core CCR (Fuaf) has a strong impact on HOPS grid.

# PSE

- General measures to avoid cross-zonal capacity reductions in the future, as per Art. 20(14)(b) of DA CCM:

PSE is taken under consideration: long-, medium- and short-term measures to prevent capacity reduction.

Generally, the main source of improvements will be grid developments, as prescribed in the Action Plan and as foreseen in the Grid Development Plan.

In medium PSE is investigating dynamic monitoring of the lines, which increase the line rating.

As the short-term measures, the main area of improvements is in the CGM grid model quality. PSE often observes cases when the F0_Core was significantly higher then 30% Fmax, or even higher than 60-70% Fmax. Hence the quality of the CGM should be closely monitored and improved. Such high level of F0 will undermine any efforts.

It should be mentioned that in many cases the reduction of the capacity due to IVA implementation was done due to specific market situation. This leads to the less flexible generation capability or unusual  flows. The generation capability are strictly connected with the redispatching potentials and in consequence with the IVA value. Additional unusual flows caused the congestions in area that was never identified as to be potentially congested.

Additionally, PSE will consider parametrization of the validation tool, potentially leading to avoiding application of low IVA values (so that IVAs will be less frequent). This is yet to be analysed and investigated.

In some cases the IVA was implemented in specific maintenance situation, this will be only temporary and additional investigation are not foreseen,

- Detailed report and action plan describing how such deviations are expected to be alleviated and solved in the future:

  - Reinforcement of the grid is included in grid development plan for the years 2025-29.

  - The new line 400 kV line Rzeszów - Khmelnytska with Ukraine may reduce flows in the congested area (the line that is most often indicated as limiting, which was previously never identified as potentially congested). The line is planed be in operation by the end this year.

  - The parametrisation might reduce frequency of IVA. Additional analysis and test will be needed to finally conclude on this, following by a decision on PSE side and necessary operators training.

  - Monitoring of CGM quality in case of F0_Core. This is a very important element, for which unfortunately PSE is not able to do much, since CGM quality results from modelling issues from power systems outside of PSE.

  - Special situation on the market influences redispatching potentials. Therefore, improvements of the fuel availability for the conventional power plant will increase redispatching potentials and in consequence value and frequency of the IVA  implementation. This however depends of the market situation and lies outside of PSE competences and authority.

# RTE

- Grid development

RTE IVAs have been done mainly from June 9th to 13th due the planned outage of Mastaing-Avelgem FR-BE interconnector, impacting Avelin-Avelgem FR-BE tieline. This maintenance has significantly increased Mastaing-Avelgem thermal limits.

A similar maintenance is ongoing on Avelin-Avelgem FR-BE interconnector since August 8th and will also lead to increased thermal limits as of November.

This increase of thermal limits will improve capacities (and will reduce application of IVAs).

- Better coordination of thermal limits with Elia

The thermal limits of FR-BE interconnectors are not the same on French and Belgium side. Current process is conservative in certain cases, and this will be adapted in September: for a given FR-BE tie-line, when the BE thermal limit is smaller than the FR thermal limit, RTE will let Elia the responsibility to limit capacities under BE thermal limits and RTE won’t consider BE thermal limits anymore.

- Individual validation improved with inclusion of topological remedial action optimizer

RTE is developing the integration of a NRAO in order to improve the inclusion of topological remedial actions in the validation step. This tool integration is expected to be done by end of 2022.

# SEPS

Expected input to be provided:

- General measures to avoid cross-zonal capacity reductions in the future, as per Art. 20(14)(b) of DA CCM

- SEPS applied IVA for the reported period for four timestamps and for same CNE (tie-line between CZ-SK) with different contingency. See table below:

| MTU (UTC) | CNEC name | CNEC ID | IVA [MW] |
| --- | --- | --- | --- |
| 11.6.2022 19:00 | [CZ-SK] Nosovice - Varin [DIR] [SK] + N-1 Lemesany - Krosno Iskrz 1 | SK_CBCO_00111 | 14 |
| 11.6.2022 19:00 | [CZ-SK] Nosovice - Varin [DIR] [SK] + N-1 Lemesany - Krosno Iskrz 2 | SK_CBCO_00112 | 14 |
| 11.6.2022 19:00 | [CZ-SK] Nosovice - Varin [DIR] [SK] + N-1 Sokolnice - Stupava | SK_CBCO_00113 | 132 |
| 12.6.2022 15:00 | [CZ-SK] Nosovice - Varin [DIR] [SK] + N-1 Sokolnice - Stupava | SK_CBCO_00113 | 80 |

- The reason for application of IVA for 11th and 12th of June was an outage of another tie-line between CZ-SK Sokolnice – Križovany which led to higher forecasted flows on the tie-line Nosovice – Varin. As at that time there were no sufficient remedial actions available, SEPS applied IVA. The short-term solution which can prevent application of IVA in case of this specific outage is not available which means that application of IVA can occur also ion the future. However, the volume and frequency of IVA is very limited and additionally there was no shadow price on that CNEC for timestamps where IVA was applied. In the future, the reinforcement of CNE is planned which should lead to increase of Imax from 1720 Ampers to 2400 Ampers in 2026. This reinforcements should help to prevent application of IVA or at least decrease the frequency of application.

# Transelectrica

In 2021, the Romanian Government decided to adopt an Action Plan pursuant to Article 15 of the Regulation (EU) 2019/943, including a linear trajectory for the yearly increase of the minimum capacity made available for cross-zonal trade until 31 December 2025.

For year 2022 the minimum capacity made available for cross-zonal trade on Romania – Hungary border (part of Core CCR) has been 41% from the transmission capacity) according to the Action Plan. For this year though, Transelectrica was granted a derogation on foreseeable grounds for maintaining operational security, thus the minimum capacity for cross-border trade remains at 33% from the transmission capacity, the same target provided in the Action Plan for the year of 2021.

In addition to being part of the Core CCR, Transelectrica is also part of SEE CCR with RO – BG border having operational processes for Day-ahead capacity calculation since June 2021 and first Intraday capacity calculation since October 2021. Moreover, there are three non-EU borders for which there is no coordinated capacity calculation.

Regarding the capacity calculation process, the Core DA CCM allows TSOs to correct cross-zonal capacity for reasons of operational security during the validation process individually and in a coordinated way according to Article 20 (1). Article 20(5) states that “each Core TSO shall validate and have the right to decrease the RAM for reasons of operational security during the individual validation. The adjustment due to individual validation is called ‘individual validation adjustment’ (IVA) and it shall have a positive value, i.e. it may only reduce the RAM. IVA may reduce the RAM only to the minimum degree that is needed to ensure operational security considering all expected available costly and non-costly RAs”.

For the period 20220609-20220630, Transelectrica applied an Individual Validation Adjustment (IVA) on the following CNEs part of the day-ahead capacity calculation process:

- OHL 220 kV Porțile de Fier – Reșița circ.1, 2;

- OHL 220 kV Reșița – Timișoara circ. 1,2;

- OHL 220 kV Urechești – Târgu Jiu Nord;

- OHL 220 kV Paroseni – Târgu Jiu Nord;

- OHL 220 kV Paroșeni – Baru Mare;

- OHL 220 kV Baru Mare – Hășdat;

- OHL 400 kV Porțile de Fier – Djerdap.

- TR 400/200 kV Porțile de Fier 1,2,3;

- OHL 220 kV Cluj Florești – Alba Iulia;

- OHL 220 kV Săcălaz – Timișoara;

Table 1. Average IVA per CNE along with the percentage of total IVA applications

| CNE | Average IVA applied [MW] | Percentage of total IVA applications [%] |
| --- | --- | --- |
| [RO-RO] Reșita – Timișoara circ. 1 [DIR] | 50.2 | 28% |
| [RO-RO] Paroșeni – Târgu Jiu Nord [OPP] | 37.3 | 16% |
| [RO-RO] Urechești – Târgu Jiu Nord [DIR] | 34.6 | 20% |
| [RO-RO] Paroșeni – Baru Mare [DIR] | 33.7 | 11% |
| [RO-RO] Baru Mare – Hășdat [DIR] | 24.1 | 7% |
| [RO-RO] Porțile de Fier – Reșița circ. 2 [DIR] | 22.9 | 4% |
| [RO-RO] Reșița – Timișoara circ. 2 [DIR] | 21.1 | 7% |
| [RO-RO] Portile de Fier – Reșița circ. 1 [DIR] | 18.5 | 1% |
| [RO-RO] TR Porțile de Fier 400/220 1 [OPP] | 12.4 | 2% |
| [RO-RO] TR Porțile de Fier 400/220 2 [OPP] | 12.4 | 2% |
| [RO-RO] TR Porțile de Fier 400/220 3 [OPP] | 10.6 | 2% |
| [RO-RO] Cluj Florești – Alba Iulia [OPP] | 7.9 | 0.2% |
| [RO-RO] Săcălaz – Timișoara [OPP] | 7.0 | 0.1% |
| [RO-RS] Porțile de Fier – Djerdap [OPP][RO] | 1.8 | 0.1% |

Figure 1. Average IVA applied per CNE during the reported period

Figure 2. Percentage of total IVA applications per CNE during the reported period

Figure 3. Average IVA per CNE and BD over the reported period

- General measures to avoid cross-zonal capacity reductions in the future, as per Art. 20(14)(b) of DA CCM;

- Detailed report and action plan describing how such deviations are expected to be alleviated and solved in the future.

- Development of the transmission grid

- Most of the cases with IVA applications are related to 220 kV critical network elements located in the southwest part of the country. The following measures are foreseen according to the National Action plan in order to increase the remaining available margin of these elements:

- New 400 kV OHL Porțile de Fier – Reșița planned for 2024;

- OHL 400 kV Resita – Timisoara – Săcălaz d.c. planned for 2025;

- OHL 400 kV Timișoara – Săcălaz – Arad d.c. planned for 2027;

- Increase the transmission capacity for the OHL in the 220 kV axis Urechesti – Targu Jiu Nord – Paroseni – Baru Mare – Hașdat.

- Increase the quality of the D2CF CGMs at CCR level

- At this moment there is no common D2CF CGMs process to be used for DA CC in all CCRs. Each CCR has its own rules on the IGMs (e.g. D2CF for Core TSOs and DACF for the rest of Continental Europe) and a net position forecast to be used when CGMs are created. As there is no common, harmonized and reliable net position or exchange forecast yet implemented in Europe, the assumptions taken by each CCR will lead to large uncertainties, potentially high overloads and operational situations where the available remedial action potential (including redispatching) is insufficient, thus leading to applying reductions on CNECs. This situation affects the power flow on the 220 kV critical network elements located in the southwest part of the country, heavily influenced by the exchanges in the SEE region where DACF files are used for the purpose of D2CF CGMs in Core CCR. A common D2CF CGM process is required for all CCRs in Continental Europe as soon as possible.

- Implementation of the coordinated validation in the Core CCR

Coordinated validation would allow TSOs to use commonly the remedial actions available throughout Core CCR, making use of the most suitable remedial action in order to secure a minimum capacity and reduce the IVA applications.

- Implementation of regional coordinated processes for security analysis

The Articles 16(4) and 16(8) of the Regulation (EU) 2019/943 refers to the implementation of the coordinated capacity calculation and security analysis at regional level to ensure a minimum capacity available for cross-zonal trade. Though the capacity calculation using flow-based method in Core CCR is now an operational process, it is not enough to comply with the minimum available capacity requirements. The results of the day-ahead capacity calculation come with a lot of uncertainties without a coordinated security analysis implemented at a regional level before real-time. Not always the internal measures and remedial actions estimated as available for day-ahead capacity calculation are available and enough to maintain the system security in real-time.

- Implementation of redispatching and countertrading processes implemented at regional level pursuant to Article 35 and 74 of Regulation (EU) 2015/1222.

The Article 16(4) of the Regulation (EU) 2019/943 stated that the redispatching and countertrading shall be used to maximize the available capacity to reach the minimum capacity provided for in Article 16(8) of the Regulation (EU) 2019/943. This process shall be coordinated and follow the implementation of cost-sharing methodology. At Core CCR level these processes are under implementation. Because of this Transelectrica applies an individual redispatching process aimed at achieving the minimum level of cross-zonal capacity as per national Action Plan. This is not always feasible due to the lack of sufficient remedial actions.

- Coordination between capacity calculation regions

Besides Core CCR, Transelectrica SA is also part of SEE CCR with RO – BG border. Furthermore, for the three non-EU borders there is no coordinated capacity calculation. The exchanges on the borders with Bulgaria, Serbia, Ukraine and Republic of Moldova are considered as fixed in coordinated capacity calculation from Core CCR.  Any deviation from these values forecasted two days ahead will create a different loading on the critical network element with risks for the operational security of the system. Because of this, lack of cross-CCR coordination becomes critical for Romania in cases with high export from the southeast part of Continental Europe towards the central area. These uncoordinated transits through Romania correlated with high generation in the wind and hydro power plants from the south part of Romania lead to increasing the power flows on the 220 kV network in the southwest part of the country. These transits create (N-1) violations in the transmission grid which cannot be addressed without coordinated remedial actions for redispatching and countertrading.

# TENNET TSO BV

In order to provide a better understanding of the results and the processes applied among German TSOs, the following are some introductory explanations:

- APG, Tennet NL and the German TSOs (DAVinCy TSOs) use a common tool for individual validation called DAVinCy (Day Ahead Validation of Capacity). In case an overload cannot be solved with the available remedial actions, DAVinCy determines the necessary Individual Validation Adjustments (IVAs) on relevant CNECs with the objective to minimise the overall capacity reduction among the six DAVinCy TSOs. This can lead to situations where an overload occurs in one control area of TSO A whereas the IVA(s) is/are applied within other control areas, e.g. of TSOs B and C.

- The application of an IVA prevents a network element from being overloaded and generally does not lead to a deviation from the minimum capacity according to Article 16(8) of regulation (EU) 2019/943 on the internal market of electricity. In case of a deviation from the minimum capacity, the compliance with the minimum capacity targets of the German action plan according to the monitoring methodology approved by the German NRA Bundesnetzagentur will be detailed within the German Compliance Monitoring Report for 2022 that will be finished in mid-2023[1].

- The short time period covered by this IVA report (09.06.2022 – 30.6.2022) does not allow for general conclusions or interpretation of the observed data. More information is expected when a broader data basis is available.

- The subsequent report for Q3 is expected to include more substantial information.

The DAVinCy TSOs plan to apply the following measures in order to minimize the application of IVAs and, as required by Art 20(14)(b) DA CCM, avoid cross-zonal capacity reductions in the future:

- Improvements within the DAVinCy tool such as cross-border redispatch between Germany and the Netherlands (already implemented in the tool, will be activated after clarification of details of the operational process) and the application of additional topological measures. Further improvements are foreseen to consider redispatch between DAVinCy TSOs’ control areas and non-DAVinCy TSOs’ control areas if redispatch potential is securely available and consideration of countertrade between Germany and Denmark.

- Continuous improvements of the input data: Many of the overloads that caused IVA application in all DAVinCy TSOs’ grids where due to erroneous data from Coreso related to AT-IT cross-border flows in the net-position forecast. This error has been fixed and measures are taken to prevent IVA application when similar problems arise.

- Improvement of process robustness through the use of redundant IT systems, plausibility checks and replacement strategies for input data, separate systems for test- and productive environment and implementation of security management.

- The recent relaxation of the fallback will lead to lower IVA in case of a failure of the DAVinCy process.

- The planned grid enforcements within the control areas of the German TSOs will relieve grid elements with overloads in the future (cf. Action Plan of Germany[2] and Network Development plan[3]).